THE  YAMAHA SYXG50 SOFTSYNTH GUIDE

Written by : Nick Howes

# This guide also contains notes for developers & system programmers.

DOWNLOAD THE LATEST VERSIONS AT WWW.YAMAHA.CO.UK

All of the data and sysex information contained within this document is related to the XG system technology used on Yamaha’s SYXG50 sound generation system. It is correct and up to date as of 1st May1997,  E&OE.

Sections of this guide are taken from the user help section of the SYXG50 installation, however…we know people too well, and they don’t read help files..they call help lines. This guide aims to try and provide a bit more fun, and some practical help too.

## Introduction.

The XG range  from Yamaha and in particular the DB50XG daughterboard has proved to be one of the most popular products that the company has released since the DX7, the unlimited power of XG coupled with the staggering low cost has guaranteed overnight success.

Now with the introduction of the MMX range of Intel microprocessors and high speed Pentium chips in general, Yamaha have been able to do what many thought to be impossible , that is to produce a fully programmable synth in software alone.

The purpose of this guide is to enable you the user to get at all of the main features of the SYXG50, and to give a non technical introduction into the world of sound  for the PC. It will also give some examples of data entry and a brief introduction into the world of system exclusive data.

The SYXG50 is based upon a technology known as Wavetable Synthesis. Wavetable synthesis is where an acoustic sample of an instrument such as a piano is recorded using a microphone, and stored in the onboard wave Rom of the card.

Wave Roms come in all shapes and sizes, with one of the most popular on the market being a paltry 1MB in size. (You know who you are !!!!!)

Yamaha have a long history in musical instruments, over 100 years in fact , so we do know a little bit (understatement of the decade !) about design when it comes to hardware and software.

Yamaha have designed the most successful synthesisers in history, and we are, by a massive margin ,the world’s largest manufacturer of musical instruments.

### FACT : THE WORLD’S MOST SUCCESSFUL SYNTHESISER, 15 YEARS ON ,IS STILL THE YAMAHA DX7

Anyway..back to waves..enough of the PR exercise.

A wave sample of an instrument  can sound very static and boring. Imagine if you will, a guitar player.(easier for females than males!).  Every time he(or she) hits the guitar string it sounds different, ever so slightly, but never the less it is different. This is where real time modifiers come in to play. The ability to change a sound in real time is very important and the SYXG50 software synth  with it’s substantial editing capabilities and effects processing,  puts all of these facilities in your grasp.

What do we mean by editing capabilities ? Well if you take for instance a guitar. To really hear the full range of sounds that a guitar can produce you would probably need the following.

1: A guitar amplifier

2: A range of effects pedals

3:A good plectrum

4: Technique (user interface only)…(sic)

The editing of a guitar sound comes from the use of the amplifier and the effects pedals. If you wanted to enhance the richness of the guitar sound, you may wish to add chorus, which simulates the sound of many instruments playing, it is in a similar way that you edit and modify SYXG50.

You may be aware of a terminology known as “bit resolution”. Many sound cards on the market today are stated as being 16bit, other claim to be 32 bit which is total nonsense as the sample resolution is still 16bit, and there has yet to be made a 32bit D/A for audio on a sound card.

The bit resolution of a device is basically referring to the number of samples taken in a given period of time of the instrument concerned.

If you take for example a pure sine wave. If you take a sample of this at a low bit resolution (eg 8 bit) then the representation of the sample will look more like a set of steps than a smooth curve.

At 16 bit resolution the number of samples per second is greatly increased, and the sample looks much more like the original waveform.

Then there is the issue of bandwidth. 44.1khz is the normal sampling frequency asscociated with the term CD quality. As the theoretical frequncy range of the human ear is around 20khz, this represents approximately double the frequncy range of the human ear. This conforms to a concept known as the Nyquist rate (sampling theory.

One of the most important contributions to sampling theory (for any signal)

is the idea of the Nyquist rate.

Sampling theory says :

If the highest frequency component in a signal is f (Hz) then the

signal should be sampled at the rate of at least 2f (Hz) for the

original signal to be completely represented by the discrete samples.

Basically, the higher the sampling rate and the higher the bit resolution, the better the sample. The trade off on this is that you tend you use bucket loads of memory with better quality sampling.

#### SO WHAT IS MIDI THEN ?

MIDI is an acronym that stands for Musical Instrument Digital Interface, which allows electronic musical instruments to communicate with each other, by sending and receiving compatible Note, Control Change, Program Change and various other types of MIDI data, or messages.

GM attempted to standardise MIDI into a set of 128 commonly used instruments, and whilst XG is fully GM compatible (if it has to be!), it comes with a lot more voices than 128.

HOW MANY ????????

Take a look at this, a feast of sounds.

The voices are  assigned into groups.

Piano

Chromatic Percussion

Organ

Guitar

Bass

Strings

Ensemble

Brass

Reed

Pipe

Synth Lead

Synth Pad

SynthEffects

Ethnic

Percussive

Sound Effects

SFX Voice

Bank Select MSB=000, LSB=Bank Number

Bank   0:(GM)

Bank   1:Key Scale Panning

Bank   3:Stereo

Bank   6:Single

Bank   8:Slow

Bank  12:Fast Decay

Bank  14:Double Attack

Bank  16:Bright

Bank  17:Bright

Bank  18:Dark

Bank  19:Dark

Bank  20:Resonant

Bank  24:Attack

Bank  25:Release

Bank  27:Reso Sweep

Bank  28:Muted

Bank  32:Detune 1

Bank  33:Detune 2

Bank  34:Detune 3

Bank  35:Octave 1

Bank  36:Octave 2

Bank  37:5th 1

Bank  38:5th 2

Bank  39:Bend

Bank  40:Tutti

Bank  41:Tutti

Bank  42:Tutti

Bank  43:Velo-Switch

Bank  45:Velo-Xfade

Bank  64:Other wave

Bank  65:Other wave

Bank  66:Other wave

Bank  67:Other wave

Bank  68:Other wave

Bank  69:Other wave

Bank  70:Other wave

Bank  71:Other wave

Bank  72:Other wave

Bank  96:Other wave

Bank  97:Other wave

Bank  98:Other wave

Bank  99:Other wave

Bank 100:Other wave

Bank 101:Other wave

Category

Piano

Program#	Bank#	Voice Name	Element

1	0	GrandPno	1

1	GrndPnoK	1

18	MelloGrP	1

40	PianoStr	2

41	Dream	2

2	0	BritePno	1

1	BritPnoK	1

3	0	E.Grand	2

1	ElGrPnoK	2

32	Det.CP80	2

40	ElGrPno1	2

41	ElGrPno2	2

4	0	HnkyTonk	2

1	HnkyTnkK	2

5	0	E.Piano1	2

1	El.Pno1K	1

18	MelloEP1	2

32	Chor.EP1	2

40	HardEl.P	2

45	VX El.P1	2

64	60sEl.P	1

6	0	E.Piano2	2

1	El.Pno2K	1

32	Chor.EP2	2

33	DX Hard	2

34	DXLegend	2

40	DX Phase	2

41	DX+Analg	2

42	DXKotoEP	2

45	VX El.P2	2

7	0	Harpsi.	1

1	Harpsi.K	1

25	Harpsi.2	2

35	Harpsi.3	2

8	0	Clavi.	2

1	Clavi. K	1

27	ClaviWah	2

64	PulseClv	1

65	PierceCl	2

Category

Chromatic Percussion

Program#	Bank#	Voice Name	Element

9	0	Celesta	1

10	0	Glocken	1

11	0	MusicBox	2

64	Orgel	2

12	0	Vibes	1

1	VibesK	1

45	HardVibe	2

13	0	Marimba	1

1	MarimbaK	1

64	SineMrmb	2

97	Balafon2	2

98	Log Drum	2

14	0	Xylophon	1

15	0	TubulBel	1

96	ChrchBel	2

97	Carillon	2

16	0	Dulcimer	1

35	Dulcimr2 	2

96	Cimbalom	2

97	Santur	2

Category

Organ

Program#	Bank#	Voice Name	Element

17	0	DrawOrgn	1

32	DetDrwOr	2

33	60sDrOr1	2

34	60sDrOr2	2

35	70sDrOr1	2

36	DrawOrg2	2

37	60sDrOr3	2

38	EvenBar 	2

40	16+2"2/3	2

64	Organ Ba	1

65	70sDrOr2	2

66	CheezOrg	2

67	DrawOrg3	2

18	0	PercOrgn	1

24	70sPcOr1	2

32	DetPrcOr	2

33	LiteOrg	2

37	PercOrg2	2

19	0	RockOrgn	2

64	RotaryOr	2

65	SloRotar	2

66	FstRotar	2

20	0	ChrchOrg	2

32	ChurOrg3	2

35	ChurOrg2	2

40	NotreDam	2

64	OrgFlute	2

65	TrmOrgFl	2

21	0	ReedOrgn	1

40	Puff Org	2

22	0	Acordion	2

32	AccordIt	2

23	0	Harmnica	1

32	Harmo 2 	2

24	0	TangoAcd	2

64	TngoAcd2	2

Category

Guitar

Program#	Bank#	Voice Name	Element

25	0	NylonGtr	1

16	NylonGt2	1

25	NylonGt3	2

43	VelGtHrm	2

96	Ukulele	1

26	0	SteelGtr	1

16	SteelGt2	1

35	12StrGtr	2

40	Nyln&Stl	2

41	Stl&Body	2

96	Mandolin	2

27	0	Jazz Gtr	1

18	MelloGtr	1

32	JazzAmp	2

28	0	CleanGtr	1

32	ChorusGt	2

29	0	Mute.Gtr	1

40	FunkGtr1	2

41	MuteStlG	2

43	FunkGtr2	2

45	Jazz Man	1

30	0	Ovrdrive	1

43	Gt.Pinch	2

31	0	Dist.Gtr	1

40	FeedbkGt	2

41	FeedbGt2	2

32	0	GtrHarmo	1

65	GtFeedbk	1

66	GtrHrmo2	1

Category

Bass

Program#	Bank#	Voice Name	Element

33	0	Aco.Bass	1

40	JazzRthm	2

45	VXUprght	2

34	0	FngrBass	1

18	FingrDrk	2

27	FlangeBa	2

40	Ba&DstEG	2

43	FngrSlap	2

45	FngBass2	2

65	ModAlem	2

35	0	PickBass	1

28	MutePkBa	1

36	0	Fretless	1

32	Fretles2	2

33	Fretles3	2

34	Fretles4	2

96	SynFretl	2

97	Smooth	2

37	0	SlapBas1	1

27	ResoSlap	1

32	PunchThm	2

38	0	SlapBas2	1

43	VeloSlap	2

39	0	SynBass1	1

18	SynBa1Dk	1

20	FastResB	1

24	AcidBass	1

35	Clv Bass	2

40	TeknoBa 	2

64	Oscar	2

65	SqrBass	1

66	RubberBa	2

96	Hammer	2

40	0	SynBass2	2

6	MelloSB1	1

12	Seq Bass	2

18	ClkSynBa	2

19	SynBa2Dk	1

32	SmthBa 2	2

40	ModulrBa	2

41	DX Bass	2

64	X WireBa	2

Category

Strings

Program#	Bank#	Voice Name	Element

41	0	Violin	1

8	SlowVln	1

42	0	Viola	1

43	0	Cello	1

44	0	Contrabs	1

45	0	Trem.Str	1

8	SlowTrStr	1

40	Susp Str	2

46	0	Pizz.Str	1

47	0	Harp	1

40	YangChin	2

48	0	Timpani	1

Category

Ensemble

Program#	Bank#	Voice Name	Element

49	0	Strings1	1

3	S.Strngs	2

8	SlowStr	1

24	ArcoStr	2

35	60sStrng	2

40	Orchestr	2

41	Orchstr2 	2

42	TremOrch	2

45	VeloStr 	2

50	0	Strings2	1

3	S.SlwStr	2

8	LegatoSt	2

40	Warm Str	2

41	Kingdom	2

64	70s Str	1

65	Str Ens3	1

51	0	Syn.Str1	2

27	ResoStr	2

64	Syn Str4	2

65	SS Str	2

52	0	Syn.Str2	2

53	0	ChoirAah	1

3	S.Choir	2

16	Ch.Aahs2	2

32	MelChoir	2

40	ChoirStr	2

54	0	VoiceOoh	1

55	0	SynVoice	1

40	SynVox2	2

41	Choral	2

64	AnaVoice	1

56	0	Orch.Hit	2

35	OrchHit2	2

64	Impact  	2

Category

Brass

Program#	Bank#	Voice Name	Element

57	0	Trumpet	1

16	Trumpet2	1

17	BriteTrp	2

32	WarmTrp	2

58	0	Trombone	1

18	Trmbone2	2

59	0	Tuba	1

16	Tuba 2	1

60	0	Mute.Trp	1

61	0	Fr.Horn	2

6	FrHrSolo	2

32	FrHorn2	1

37	HornOrch	2

62	0	BrasSect	1

35	Tp&TbSec	2

40	BrssSec2	2

41	HiBrass	2

42	MelloBrs	2

63	0	SynBras1	2

12	QuackBr	2

20	RezSynBr	2

24	PolyBrss	2

27	SynBras3	2

32	JumpBrss	2

45	AnaVelBr	2

64	AnaBrss1	2

64	0	SynBras2	1

18	Soft Brs	2

40	SynBras4	2

41	ChorBrss	2

45	VelBras2	2

64	AnaBras2	2

Category

Reed

Program#	Bank#	Voice Name	Element

65	0	SprnoSax	1

66	0	Alto Sax	1

40	Sax Sect	2

43	HyprAlto	2

67	0	TenorSax	1

40	BrthTnSx	2

41	SoftTenr	2

64	TnrSax 2	1

68	0	Bari.Sax	1

69	0	Oboe	2

70	0	Eng.Horn	1

71	0	Bassoon	1

72	0	Clarinet	1

Category

Pipe

Program#	Bank#	Voice Name	Element

73	0	Piccolo	1

74	0	Flute	1

75	0	Recorder	1

76	0	PanFlute	1

77	0	Bottle	2

78	0	Shakhchi	2

79	0	Whistle	1

80	0	Ocarina	1

Category

Synth Lead

Program#	Bank#	Voice Name	Element

81	0	SquareLd	2

6	Square 2	1

8	LMSquare	2

18	Hollow	1

19	Shmoog  	2

64	Mellow	2

65	SoloSine	2

66	SineLead	1

82	0	Saw.Lead	2

6	Saw 2   	1

8	ThickSaw	2

18	DynaSaw	1

19	DigiSaw	2

20	Big Lead	2

24	HeavySyn	2

25	WaspySyn	2

40	PulseSaw	2

41	Dr. Lead	2

45	VeloLead	2

96	Seq Ana	2

83	0	CaliopLd	2

65	Pure Pad	2

84	0	Chiff Ld	2

64	Rubby	2

85	0	CharanLd	2

64	DistLead	2

65	WireLead	2

86	0	Voice Ld	2

24	SynthAah	2

64	VoxLead	2

87	0	Fifth Ld	2

35	Big Five	2

88	0	Bass &Ld	2

16	Big&Low 	2

64	Fat&Prky	2

65	SoftWurl	2

Category

Syhth Pad

Program#	Bank#	Voice Name	Element

89	0	NewAgePd	2

64	Fantasy2	2

90	0	Warm Pad	2

16	ThickPad	2

17	Soft Pad	2

18	SinePad	2

64	Horn Pad	2

65	RotarStr	2

91	0	PolySyPd	2

64	PolyPd80	2

65	ClickPad	2

66	Ana Pad	2

67	SquarPad	2

92	0	ChoirPad	2

64	Heaven2 	2

66	Itopia	2

67	CC Pad	2

93	0	BowedPad	2

64	Glacier	2

65	GlassPad	2

94	0	MetalPad	2

64	Tine Pad	2

65	Pan Pad	2

95	0	Halo Pad	2

96	0	SweepPad	2

20	Shwimmer	2

27	Converge	2

64	PolarPad	2

66	Celstial	2

Category

Synth Effects

Program#	Bank#	Voice Name	Element

97	0	Rain	2

45	ClaviPad	2

64	HrmoRain	2

65	AfrcnWnd	2

66	Caribean	2

98	0	SoundTrk	2

27	Prologue	2

64	Ancestrl	2

99	0	Crystal	2

12	SynDrCmp	2

14	Popcorn	2

18	TinyBell	2

35	RndGlock	2

40	GlockChi	2

41	ClearBel	2

42	ChorBell	2

64	SynMalet	1

65	SftCryst	2

66	LoudGlok	2

67	XmasBell	2

68	VibeBell	2

69	DigiBell	2

70	AirBells	2

71	BellHarp	2

72	Gamelmba	2

100	0	Atmosphr	2

18	WarmAtms	2

19	HollwRls	2

40	NylonEP	2

64	NylnHarp	2

65	Harp Vox	2

66	AtmosPad	2

67	Planet	2

101	0	Bright	2

64	FantaBel	2

96	Smokey	2

102	0	Goblins	2

64	GobSyn	2

65	50sSciFi	2

66	Ring Pad	2

67	Ritual	2

68	ToHeaven	2

70	Night	2

71	Glisten	2

96	BelChoir	2

103	0	Echoes	2

8	EchoPad2	2

14	Echo Pan	2

64	EchoBell	2

65	Big Pan	2

66	SynPiano	2

67	Creation	2

68	Stardust	2

69	Reso Pan	2

104	0	Sci-Fi	2

64	Starz	2

Category

Ethnic

Program#	Bank#	Voice Name	Element

105	0	Sitar	1

32	DetSitar	2

35	Sitar 2 	2

96	Tambra	2

97	Tamboura	2

106	0	Banjo	1

28	MuteBnjo	1

96	Rabab	2

97	Gopichnt	2

98	Oud	2

107	0	Shamisen	1

108	0	Koto	1

96	T. Koto	2

97	Kanoon	2

109	0	Kalimba	1

110	0	Bagpipe	2

111	0	Fiddle	1

112	0	Shanai	1

64	Shanai2	1

96	Pungi	1

97	Hichriki	2

Category

Percussive

Program#	Bank#	Voice Name	Element

113	0	TnklBell	2

96	Bonang	2

97	Gender	2

98	Gamelan	2

99	S.Gamlan	2

100	Rama Cym	2

101	AsianBel	2

114	0	Agogo	2

115	0	SteelDrm	2

97	GlasPerc	2

98	ThaiBell	2

116	0	WoodBlok	1

96	Castanet	1

117	0	TaikoDrm	1

96	Gr.Cassa	1

118	0	MelodTom	2

64	Mel Tom2	1

65	Real Tom	2

66	Rock Tom	2

119	0	Syn.Drum	1

64	Ana Tom	1

65	ElecPerc	2

120	0	RevCymbl	1

Category

Sound Effects

Program#	Bank#	Voice Name	Element

121	0	FretNoiz	2

122	0	BrthNoiz	2

123	0	Seashore	2

124	0	Tweet	2

125	0	Telphone	1

126	0	Helicptr	1

127	0	Applause	1

128	0	Gunshot	1

Bank Select  MSB=064, LSB=000

SFX Voice

MSB=064

Program#	LSB=000	Element

1	CuttngNz	1

2	CttngNz2	2

3	  ----

4	Str Slap	1

5	  ----

6	  ----

7	  ----

8	  ----

9	  ----

10	  ----

11	  ----

12	  ----

13	  ----

14	  ----

15	  ----

16	  ----

17	Fl.KClik	1

18	  ----

19	  ----

20	  ----

21	  ----

22	  ----

23	  ----

24	  ----

25	  ----

26	  ----

27	  ----

28	  ----

29	  ----

30	  ----

31	  ----

32	  ----

33	Rain	1

34	Thunder	1

35	Wind	1

36	Stream	2

37	Bubble	2

38	Feed	2

39	  ----

40	  ----

41	  ----

42	  ----

43	  ----

44	  ----

45	  ----

46	  ----

47	  ----

48	  ----

49	Dog	1

50	Horse	1

51	Bird 2	1

52	  ----

53	  ----

54	  ----

55	Ghost	2

56	Maou	2

57	  ----

58	  ----

59	  ----

60	  ----

61	  ----

62	  ----

63	  ----

64	  ----

65	Tel.Dial	1

66	DoorSqek	1

67	Door Slam	1

68	Scratch	1

69	Scratch 2	2

70	WindChm	1

71	Telphon2	1

72	  ----

73	  ----

74	  ----

75	  ----

76	  ----

77	  ----

78	  ----

79	  ----

80	  ----

81	CarEngin	1

82	Car Stop	1

83	Car Pass	1

84	CarCrash	1

85	Siren	2

86	Train	1

87	Jetplane	2

88	Starship	2

89	Burst	2

90	Coaster 	2

91	SbMarine	2

92	  ----

93	  ----

94	  ----

95	  ----

96	  ----

97	Laughing	1

98	Scream	1

99	Punch	1

100	Heart	1

101	FootStep	1

102	  ----

103	  ----

104	  ----

105	  ----

106	  ----

107	  ----

108	  ----

109	  ----

110	  ----

111	  ----

112	  ----

113	MchinGun	1

114	LaserGun	2

115	Xplosion	2

116	FireWork	2

117	  ----

118	  ----

119	  ----

120	  ----

121	  ----

122	  ----

123	  ----

124	  ----

125	  ----

126	  ----

127	  ----

128	  ----

---- : No Sound

##### MIDI Messages Received by the S-YXG50

The S-YXG50 is controlled by various types of incoming MIDI messages which automatically determine play mode, select MIDI channels, Voices and effects, change parameter values, and of course play the Voices specified for the various Parts complete with all the subtle nuances and powerful dynamics of expression that the composer originally intended.

Below is an explanation of the various types of MIDI messages which the S-YXG50 can receive.

##### Key On/Key Off

Key On/Key Off messages, also called Note messages, tell the S-YXG50 which notes to play, the Velocity value (depending on how hard the keys are struck) at which to play them, and how long to play them i.e., when to start (On) and stop (Off) playing each note.

##### Control Change

Control Change messages let you select a Voice bank, control volume, panning, modulation, portamento time, brightness and various other controller parameters, through specific Control Change numbers which correspond to each of the various parameters.

Program Change

Program Change messages tell the S-YXG50 which Voice to select for each Part. You can insert Program Change messages at any desired location in a song. Combining Bank Select

numbers let you select various Voices from the hundreds of Voices available in the S-YXG50.

Pitch Bend

Pitch Bend messages are continuous controller messages that allow the pitch of designated notes to be raised or lowered by a specified amount over a specified duration.

Channel Aftertouch

Channel Aftertouch is a pressure sensing function which lets you control various functions by how hard you press the keys, over the entire channel.

##### Polyphonic Aftertouch

Polyphonic Aftertouch is a pressure sensing function which lets you control various functions by how hard you press the keys, for each individual key.

System Exclusive

System Exclusive messages control various functions of the S-YXG50, including master volume and master tuning, play mode (XG or TG300B), effect type and various other parameters

specifically related to the S-YXG50.

##### Decimal - Hexadecimal Conversion Chart

Many MIDI messages listed in the “MIDI Data Format” section, are expressed in hexadecimal numbers.

The chart below lists the corresponding decimal number for each hexadecimal number. (Hexadecimal numbers may include the letter “H” as a suffix.)

Dec Hex Dec Hex Dec Hex Dec Hex

0 00 32 20 64 40 96 60

1 01 33 21 65 41 97 61

2 02 34 22 66 42 98 62

3 03 35 23 67 43 99 63

4 04 36 24 68 44 100 64

5 05 37 25 69 45 101 65

6 06 38 26 70 46 102 66

7 07 39 27 71 47 103 67

8 08 40 28 72 48 104 68

9 09 41 29 73 49 105 69

10 0A 42 2A 74 4A 106 6A

11 0B 43 2B 75 4B 107 6B

12 0C 44 2C 76 4C 108 6C

13 0D 45 2D 77 4D 109 6D

14 0E 46 2E 78 4E 110 6E

15 0F 47 2F 79 4F 111 6F

16 10 48 30 80 50 112 70

17 11 49 31 81 51 113 71

18 12 50 32 82 52 114 72

19 13 51 33 83 53 115 73

20 14 52 34 84 54 116 74

21 15 53 35 85 55 117 75

22 16 54 36 86 56 118 76

23 17 55 37 87 57 119 77

24 18 56 38 88 58 120 78

25 19 57 39 89 59 121 79

26 1A 58 3A 90 5A 122 7A

27 1B 59 3B 91 5B 123 7B

28 1C 60 3C 92 5C 124 7C

29 1D 61 3D 93 5D 125 7D

30 1E 62 3E 94 5E 126 7E

31 1F 63 3F 95 5F 127 7F

#### WHAT EXACTLY IS SYXG50

SYXG50 is a software synth, and here is a brief history of software synths.

Several years ago, a company by the name of  SeerSystems were approached by Intel to develop a new technology for them based upon the principles of  software synthesis. This had been around for many years in research labs and academic institutions such as IRCAM, but Intel wanted to experiment with the commercial possibilites of the system. The original Seer model, whilst functional, didn’t really stretch the  boundaries of modern synthesis.

##### The concept behind software synthesis falls currently into 3 categories

1: Software wavetable

This is the method currently employed by the Yamaha softsynth. Wavetable sound cards have dominated the market place now for the last 2-3 years, with popular models being those from the likes of Creative Labs. These cards have however lacked most of the professional realtime editing facilities that keyboard programmers are used to, and also have very small wave Rom sets.

In late 1995 Yamaha started testing the DB50XG. This is a wavetable daughterboard, and coming from a company that leads the world in musical instrument development, it featured one of the most powerful synth engines ever devised for the PC. It scooped over 20 awards in the UK, and is still one of the most popular products Yamaha makes today. However with the advancement of PC technology, it became apparent, that using clever coding, this technology from the DB50XG could be emulated in software, using the power of the Intel Pentium series of processors to simulate many of the functions of the hardware found on the DB50XG. It was with this in mind that SYXG50 was conceived. Early versions of the Yamaha softsynth engine were much the same as the other systems on the market, GM only, no real editing ability, and limited effects. Yamaha , not being content with this expanded rapidly with both the voice data set and with the overall capabilities of SYXG50, until today when we can bring the most powerful softsynth in the world to life.

2: Waveguide/Physical modeling.

This is the technique used partly on the new AWE64, and whilst it uses CPU power also, it is still quite a basic model in use. Waveguide is in essence a bi-directional delay line, that can replicate string instruments reasonably well. It is a nice method of sound generation. Basic waveguide as used by the AWE though cannot realistically impersonate sounds in the way the the advanced SVA techniques used on the Yamaha VL range can. SVA uses a much more complex algorithm set to generate sound than has ever been used before. We can only wait and see what future developments from Yamaha will take place in this field.

3: Rendering of Sounds (such as CSOUND)

Many articles have been written on this powerful synthesis system, still used mainly in academia.  All you need to do is hunt on the Internet for CSOUND to come up with a wealth of info, that this brief guide could no more than touch upon.

#### WHAT DO I NEED TO RUN THIS BEAST THEN ?

CPU	Intel Pentium 166MHz or higher (AMD K6 CPU will also function)

(MMX ™ Technology Pentium Processor  is recommended)

OS	Windows 95 (OSR2 or standard)

RAM	More than 16MB (32MB highly recommended)

Sound Processing	16-bit stereo playback (CODEC on Soundcard)

CYRIX NOTES :

Yamaha has determined that the design of the 6X86 Series from Cyrix Corp., means that it is very difficult for the Yamaha S-YXG50 to support the Cyrix 6X86 without compromising the system performance of Windows95. The S-YXG50 driver software relies on the efficient execution of a large number of precise, high-speed,floating point multiplication instructions. Unfortunately, the Cyrix 6X86 FPU (floating point unit) does not - unlike the FPUs of other manufacturers - provide pipelining for floating point instructions, thus making it impossible to group multiplications together. Due to the lack of this feature on the Cyrix 6X86, the S-YXG50 would, if activated, consume an unacceptable percentage of the 6X86 CPU cycles, degrading other functions (such as video and graphics) and ultimately providing an unacceptable overall system performance. To avoid this outcome, the S-YXG50 should not be active when used with a Cyrix 686 machine. The end result is that Cyrix users should really try to opt for one of the Yamaha hardware XG solutions such as the multi award winning DB50XG, or the SW60XG. The aforementioned problem not only occurs with the Yamaha S-YXG50 but also with many other manufacturers software wavetable systems.

The latest version of the SXG50 will in fact cause many cyrix machines to completely, hang. I found this out thorugh painful experience (my 686 P200+ at home went over more times than a drunk on a unicycle after a heavy night out ). So if you  have a 686, then you have a few courses of action.

1: Do what I did , and get a P200MMX ( and a tasty new motheboard)

2: Download SYG20 (this works fine)

3: Get a DB50XG (full on XG entertainment)

4: Wait until we try to open up the CYRIX a bit more , and try to get this sorted out.

Yamaha will endevour to support future products by Cyrix and other CPU manufacturers along with the standard Intel range of CPU's, as we feel that all users, regardless of their CPU should be able to benifit from the Yamaha XG sound set.

As mentioned before the SYXG50 is a wavetable upgrade in software, it is not designed as a stand alone device to fit into a PC as it requires a host  (such as a Creative Labs Sound Blaster 16/32/64), for wave playback. The thing is that practically every PC on the planet has a sound card of some description, from cheap £20 SB clones to state of the art digital i/o systems. As long as it can play back at a minimum of 16 bit (and 44.1 K), then the Yamaha SYXG50 will work with it.

The SYXG50 uses for the first time ever on a software wavetable set  card, multiple DSPs. A DSP is a digital signal processor or effects unit . If you take for example, one the most common sound cards on the market, the AWE32, this card has a limit of 3 effects types... chorus , reverb and surround sound. Not really relevent for someone like a guitar player !!!. Unlike all other cheap sound cards (and indeed expensive ones) the SYXG50 can run over 40 different effects from 3 independent processors at the same time . This is not a misprint ,up to 3 independent effects processors running simultaneously , and with any combination of up to 42 different effects types ranging from DISTORTION, FLANGER , WAH WAH, EQ, DELAY, REVERB , PHASER, ER PLATES ...etc etc. These effects are all editable and controllable in real-time with up to 16 parameters per effect. The advantage of effects processing can be heard in any commercial music track available today. Basic effects such as reverb can enhance any sound card, but to hear the roar of a guitar you need guitar effects such as distortion and compression... try finding those on any other non XG device. The SYXG50 has a massive  676 feature packed instrument sounds, all of which can be fully edited down to subtleties such as pitch envelopes, filter cutoff and resonance. The SYXG50 has a full 24db per octave 4 pole fully resonant set of filters, again a totally unique feature to this system. The SYXG50 has 21 drum kits, yes .. drum kits, each drum kit sound and there are up to 63 on each kit, can be fully programmed down to individual sample level, and then passed through any 3 of the 64 realtime dynamic effects.

The system  is fully GM level 1 compliant, and XG compatible, (and TG300B also).

The most common question is “how do I get to all of this power” , by using this easy guide, you should have of the answers you need.

The second most common question is “what is it really, and what is it compatible with ?”

The SYXG50 uses a software emulation of one of Yamaha’s most powerful synth engines , an engine that can be found in many of the popularYamaha XG units such as .

MU50       MU80  ,QS300 ,QY70 , SW60XG and   MU10

## CHAPTER 1 : Installing SYXG50

1	Insert the disk into the computer’s disk drive.

2	Double-click on the YAMAHA folder to open it, then double-click the “Setup.exe” file in the S-YXG50 folder.

3	Restart your computer when the installation is complete.

## When the S-YXG50 is installed the following commands will be added to the “system.ini” file.

[386Enh] section

device =vswp.vxd

[Driver] section

midixx =sXGb.drv

wavexx =sXGb.drv

(“xx” is the number appended to the WAVE and MIDI drivers prior to installation)

## Multimedia Property Settings for the S-YXG50

MIDI	Select the YAMAHA SXG50 DRIVER.

AUDIO	Normally a hardware wave device driver would be selected for the audio section. If MIDI and wave data is to be played simultaneously with an application that does not support Direct Sound, also select the SXG50 DRIVER for the audio section. When Direct Sound is to be used select the hardware wave device driver.

## Chapter 2 : Configuring the SYXG50 to run

When the S-YXG50 is installed the XG Synth Driver icon will appear in the control panel.

Double-click “My Computer”, then double-click the XG Synth Driver icon in the control panel folder to access the settings display panel.

When this is open you will see the following controls.

##### Effect

All three effects are on by default. Each effect can be turned on or off as required.

##### Sampling Rate

The sampling rate setting determines the playback audio quality. The higher the sampling rate the higher the sound quality. Since higher sampling rates place greater demand on the CPU’s processing capabilities, select a sampling rate that is appropriate for your computer.

The default sampling rate is 22 kHz.

Some irregularities may occur during MIDI file playback when the sampling rate is set to 44 kHz, this depend on the speed of your CPU though, remember  the faster the better.

##### CPU Load

Specifies the amount of CPU capacity the software synthesizer may use. The default setting is 70%. The lower the setting the fewer the number of simultaneous notes the S-YXG50 will be able to produce. It is worth while experimenting with this though.

##### Polyphony

Specifies the total number of voices to be played at one time using the soft synthesizer. The default setting is 32-note polyphony. Up to 128 notes can be selected depending on the computer CPU you use.

##### Direct Sound

The default setting is “Off”.

Turn this parameter on when using the S-YXG50 with Direct Sound compatible games.

When the Direct Sound parameter is turned “On”, the multimedia property audio section should be set to hardware wave driver.

Direct Sound should be turned “Off” when a MIDI playback application is used.

[OK] Button

Click this button to save the settings and exit from the settings display.

[Cancel] Button

Click this button to exit from the settings display without saving the settings.

The S-YXG50 has 128-note polyphony and is 16-Part multitimbral, which means it can play up to 16 different instrument Parts (one Part per each of the 16 MIDI channels), each Part having its own Voice. Up to 16 different Voices can be sounded simultaneously.

The S-YXG50 features both XG and TG300B performance modes. Normally the S-YXG50 plays in XG mode. However, since it automatically recognizes which mode to select based on

incoming MIDI data, you can easily change the mode by MIDI System Exclusive messages that you program in sequencer data, at the head of your composition.

Please note the following:

- Starting up your computer will automatically initialize all S-YXG50 settings, or parameter values.

- Since the S-YXG50 requires about half a second to change modes when it receives a mode change message, be sure to insert at least one blank measure at the top in order to allow enough time for the mode to change before your actual music data begins.

XG Mode

In XG mode the S-YXG50 will play XG-compatible multitimbral data, as well as multitimbral data created for the GM System Level 1 format.

In XG mode the S-YXG50 can:

- Play up to 16 Parts.

- Choose from 480 Normal Voices and 11 Drum Voices.

##### TG300B Mode

In TG300B mode the S-YXG50 will play multitimbral music data created for TG300B-compatible tone generators. TG300B mode also provides compatibility with the GM System Level 1 format.

In TG300B mode the S-YXG50 can:

- Play up to 16 Parts.

- Choose from 579 Normal Voices and 10 Drum Voices.

- The easiest way to get at TG300B mode is to

- Download SCEDIT from Gary Gregson’s home page.

- Use this guide.

##### Voices and Elements

A S-YXG50 Voice can be made up of one or two elements (i.e., sounds), depending on the complexity of the Voice. Many Voices are made up of only one element. Two-element Voices

are made up of two one-element Voices, and may be configured in a variety of ways.

The number of elements in use determines the maximum number of simultaneous notes (polyphony) which the S-YXG50 can play at any given time, depending on the amount of incoming MIDI Note data. For details about the number of elements used in each Voice, see the Normal Voice Lists (XG Normal Voice List, TG300B Normal Voice List).

##### Normal Voices and Drum Voices

The S-YXG50 has two types of Voices---Normal Voices and Drum Voices. (In this Help File, in general the word Voice refers to a Normal Voice.)

The distinction between a Normal Voice and a Drum Voice is as follows:

- A Normal Voice is simply a pitched Voice which can be played on a musical scale from low to high, such as a piano or trumpet. The S-YXG50 has 676 Normal Voices.

- A Drum Voice is a complete set of drum and other percussion sounds, each sound having a fixed pitch. Each sound is assigned to a specific MIDI Note number (which also corresponds to a key on a MIDI keyboard). The S-YXG50 has 21 Drum Voices.

##### Maximum Polyphony

The S-YXG50 can play a maximum of 128 notes polyphony at once. However, the actual number of notes that will play at any given time is determined by the number of elements in use across the 16 Parts.

For example, if you use only one-element Voices, you can achieve the full 128 notes maximum polyphony. If you use one or more two-element Voices, however, maximum polyphony will be reduced accordingly.

The S-YXG50 is a last-note-priority tone generator, which means that if it receives more than 128 notes of MIDI Note data at any time, earlier (first) notes will automatically cut off to accommodate the most recent (last) incoming notes.

##### Part Priority

Each of the S-YXG50’s 16 Parts corresponds to each of the 16 MIDI channels (1-16). If incoming Note data exceeds the maximum polyphony, the S-YXG50 will prioritize which Parts are played first, in the following order, from higher priority to lower:

- Channel 10 (Drum Part), 1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 12, 13, 14,15, 16.

Therefore, if you are creating your own music data using sequencer software, you should assign your most important Parts (such as melody and bass) to higher priority MIDI channels (i.e., lower MIDI channel numbers) in order to preserve the integrity of your composition. Note that with the GM System Level 1, the Drum Part is always assigned to MIDI channel 10.

##### Element Reserve

The S-YXG50 has an Element Reserve feature that lets you reserve a specified number of notes for certain Parts, in order to keep notes from being “stolen” from those Parts by other Parts if incoming MIDI Note data exceeds maximum available polyphony.

For example, if you specify an Element Reserve value of “10” for Part 1, then Part 1 will always keep 10 elements for itself. You can set Element Reserve values with MIDI System Exclusive messages (XG Native Parameter Change and <Table 1 - 4>).

##### Selecting Voices

The S-YXG50 not only contains the 128 basic GM Voices and GM drum set, but many variation Voices as well—to give you access to a total of 676 Normal Voices and 21 Drum Voices.

In XG mode, the basic 128 GM Voices can be accessed by selecting Program numbers 1-128. Other Voices can be accessed by selecting both bank numbers and Program Change numbers. The Voice bank can be selected via MIDI Control Change Bank Select (MSB and LSB) messages. In XG mode, the MSB value determines the Voice type (Normal, Drum), while the LSB value actually selects the bank (excluding the SFX bank).

When the S-YXG50 is in TG300B mode, the Voice banks can be selected with appropriate MSB numbers, as LSB is fixed.

Lists of all available Voices along with bank and program numbers :

XG Normal Voice List, TG300B Normal Voice List, XG Drum Voice List, TG300B Drum Voice List.

Note that the 128 MIDI Program Change numbers consist of 0 through 127, whereas the 128 S-YXG50 program numbers consist of 1 through 128. Depending on the sequencing hardware and software you use, you may have to convert the S-YXG50 program numbers to the appropriate Program Change numbers.

- For more information : Bank Select, Program Change messages

##### Effect Types

The S-YXG50 features dozens of extremely versatile digital effects generated by Yamaha’s advanced Digital Signal Processing (DSP) technology—which add a completely new dimension to your computer’s sound.

There are three distinct effect types, or effect sections, each of which include a variety of individual effects. There are 8 Reverb type effects, 8 Chorus type effects, and 36 Variation type effects. For a complete list of effects, see the Effect Type List.

Reverb, Chorus and Variation effect types are configured, or routed, in one of two ways—to be either a System Effect or an Insertion Effect. The difference is as follows:

SYSTEM EFFECT	Applies the designated effect to all 16 Parts.

INSERTION EFFECT	Applies the designated effect to only one specific Part.

Reverb and Chorus effect types are dedicated System Effects, and therefore are applied to the overall “mix”. The Variation effect type, however, can be configured as either a System Effect or an Insertion Effect. To designate effect types and parameter values via MIDI messages, see XG Native Parameter Change.

## Setting up with games

Most PC games have a setup utility. This allows you to setup things such as the keyboard layout, and the joystick port. It also allows you to set up the sound device drivers for the game.

Most games now have the option for Special FX and music playback. With special effects you use the standard Sound Blaster setup of IRQ5 base address 220H and DMA channel 1. With the SYXG50 you  just apply the SYXG50 driver in your Windows control panel to be the default MIDI device. Don’t forget that it is designed for Win95 games only, and not DOS.

A good one to try is DOOM95, on a P200MMX it flies !

## Troubleshooting:

##### No Sound

Check that the driver is installed and that the wave playback part of your soundcard mixer is set to a resonably high level.

Also try to make sure that no other audio devices are hogging the Audio playback side of your host soundcard.

Have you checked the direct sound settings in accordance with the notes above ?

#### THE PRINCIPLES OF SOUND

If you imagine you had just bought a compact disc player, and wanted to hear it at it’s best. You would not plug it into a cheap and nasty set of speakers, you would plug it into a top of the range hifi system. The same goes with the SYXG50. If you use it in conjunction with a cheap Sound Blaster or compatible card, all of the noise and hiss asscociated with the Sound Blaster card will tarnish the sound playback of  the SYXG50. This is not to say that it will not work, but why put 2 star petrol into your Ferrari, when you can have 4 star ? (A nice little analogy).

As mentioned previously , the SYXG50 uses sampled sounds or “wavetables” to generate the complex tones that you hear. The sound produced can be broken down into individual components, all of which are important, and all of which can have either dramatic or subtle effects on the overall tone.

SOUND QUALITY TECHNICAL TERM PARAMETER

OVERALL TIMBRE HARMONIC CONTENT WAVEFORM

BRIGHTNESS AMP OF HF HARMONICS FILTER CUTOFF

VOLUME CHANGES DYNAMIC AMPLITUDE VCA ENVELOPE

AND NOW…..THE FUN BIT !!!!!….ANORAKS AT THE READY……O.K…LETS GO !

#### CHAPTER 3 : SYSTEM EXCLUSIVE DATA

After looking at the XG specs you are probably thinking one of 3 things.

1 : Where can I sign up for a degree course in ASTROPHYSICS

2:  HELP !!!!!!

3:  I had better call YAMAHA and ask them to explain what it all means.

The world of sysex is very daunting at first. We have all been though it here at Yamaha, and once you have grasped the main concepts it is a very rewarding way of working.

Before I go into some typed examples I will try to explain the fundamentals of SYSEX data.

SYSEX is a way of talking to the SYXG50 at  one of it’s most basic levels. The SYXG50 although just a piece of software, really in principle is a computer, and to talk to a computer you need to learn how to program it. As with computers, application packages are designed to help make things easier to use, and the excellent XGEDIT by Gary Gregson, is one such program. If however you wish to modify things at a real base level then sysex is the way to do it.

Sysex is comprised of a series of data bytes, they have a specific order and mean specific things when typed in that order. When typed in incorrectly they will wither do nothing at all or will give an unexpected result. This is why I have typed out all of the major control parameters

The data bytes mentioned are in what is known as Hexadecimal. This is a 2 byte alpha numeric format which basically is a number anywhere between 0 and 255 (eg 00 = 00    OA=10  7F= 127)

The data is entered using the sysex editor or event list editor in your sequencer (more of which is discussed later)  in the following way.

The first data byte (F0) is known as the start of sysex

Byte 2  (43) is the Yamaha ID code

Byte 3 (10) is the device number for SYXG50

The next 4 data bytes correspond to the command specific data. If you imagine these data bytes are saying “ok now I want to change a parameter such as filter cutoff and give it a value of 127)

then the complete data stream would be as follows

#### F0 43 10 4C 08 00 18 7F F7

THE 7F IS THE VALUE OF 127, THE (08 00 18) IS TELLING THE SYXG50 THAT YOU WISH TO ACCESS THE FILTER CUTOFF PARAMETER.

In specific sequencers please refer to your sequencer manual for information relating to the input of sysex data

# Anyway on with the show..here is the worked example guide.

Below is a list of standard MIDI controller numbers used by XG.

Controller Name Number Range

Modulation Wheel 1 0-127

Breath Controller 2 0-127

Early DX7 Aftertouch 3 0-127

Foot Controller 4 0-127

Portamento Time 5 0-127

Data Slider 6 0-127

Main Volume 7 0-127

Balance 8 0-127

Pan 10 0-127

Expression 11 0-127

General Purpose #1 16 0-127

General Purpose #2 17 0-127

General Purpose #3 18 0-127

General Purpose #4 19 0-127

LSB’s for #’s 0-31 32-63 0-127

Sustain Pedal 64 0 or 127

Soft Pedal 67 0 or 127

Hold 2 69 0 or 127

filter & harmonic content 74-71 0-127 (The ones you will probably tweak the most)

General Purpose #5 80 0 or 127

General Purpose #6 81 0 or 127

General Purpose #7 82 0 or 127

General Purpose #8 83 0 or 127

Tremelo Depth 92 0-127

Chorus Depth 93 0-127

Celeste Depth 94 0-127

Phase Depth 95 0-127

Data Increment 96 0 or 127

Data Increment 97 0 or 127

Non Reg. Parameter MSB 98 0-127

Non Reg. Parameter LSB 99 0-127

Reg. Parameter MSB 100 0-127

Reg. Parameter LSB 101 0-127

reset all controllers 121 127

Local Control On/Off 122 0 or 127

All Notes Off 123 0

Omni Off 124 0

Omni On 125 0

Mono On (0=omni off) 126 0-16

Poly On 127 0

#### CHAPTER 4: THE COMPLETE SYSEX GUIDE TO THE YAMAHA SYXG50

The following data will allow access to most major parameters, where a value of nn is inserted this represents the data byte.The data can either have a value of zero or one if it relates to an on/off switch command, or can have a value up to 127 ( in hex=7f) if it relates to a variable parameter such as filter cut-off. A list of decimal to hex conversions can be found in this guide manual. These parameters when entered into the event list in most sequencers should not be at the same tick or midi time, also to enable these to work a GM on message must be sent, and 200 milliseconds later the XG on message, both of which are in this guide.

Event list editing and the entry of the below parameters should always be performed after the card has been set into the relevent mode (XG or TG300B).

FILTER CUTOFF     F0 43 10 4C 08 00 18 NN F7     (0-127)

THIS CAN ALSO BE DONE WITH CONTROLLERS & NRPN’S (NON REGISTERED PARAMETER NUMBERS)

RESONANCE F0 43 10 4C 08 00 19 NN F7

ATTACK TIME F0 43 10 4C 08 00 1A NN F7

DECAY TIME F0 43 10 4C 08 00 1B NN F7

RELEASE TIME F0 43 10 4C 08 00 1C NN F7

PEG INITLVL F0 43 10 4C 08 00 69 NN F7

PEG ATAK TIME F0 43 10 4C 08 00 6A NN F7

PEG REL LVL F0 43 10 4C 08 00 6B NN F7

PEG REL TIME F0 43 10 4C 08 00 6C NN F7

VIBRATO RATE F0 43 10 4C 08 00 15 NN F7

VIBRATO DEPTH F0 43 10 4C 08 00 16 NN F7

VIBRATO DELAY F0 43 10 4C 08 00 17 NN F7

DETUNE F0 43 10 4C 08 00 09 08 NN F7

PART MODE F0 43 10 4C 08 00 07 NN F7

MONO POLY F0 43 10 4C 08 00 05 NN F7

VEL SENS DEPTH F0 43 10 4C 08 00 0C NN F7

VEL SENS OFFSET F0 43 10 4C 08 00 0D NN F7

NOTE LIMIT F0 43 10 4C 08 00 0F NN F7

NOTE LIMIT HIGH F0 43 10 4C 08 00 10 NN F7

PORAMENTO SW F0 43 10 4C 08 00 67 00 F7

PORTAMENTO TIME F0 43 10 4C 08 00 68 NN F7

VELOCITY LIMIT F0 43 10 4C 08 00 6D NN F7

VEL LIMIT HIGH F0 43 10 4C 08 00 6E NN F7

PITCH BEND CNTRL F0 43 10 4C 08 00 23 NN F7

MW LFO MOD F0 43 10 4C 08 00 20 NN F7

VARIATION SEND F0 43 10 4C 02 01 5B NN F7

EQ TYPE F0 43 10 4C 02 40 00 NN F7

80 HZ F0 43 10 4C 02 40 01 NN F7

500 HZ F0 43 10 4C 02 40 05 NN F7

1.0 KHZ F0 43 10 4C 02 40 09 NN F7

4.0 KHZ F0 43 10 4C 02 40 0D NN F7

8.0 KHZ F0 43 10 4C 02 40 11 NN F7

REVERB TYPE F0 43 10 4C 02 01 00 01 NN F7

REVERB TIME F0 43 10 4C 02 01 02 NN F7

DIFFUSION F0 43 10 4C 02 01 03 NN F7

INIT DELAY F0 43 10 4C 02 01 04 NN F7

HPF CUTOFF F0 43 10 4C 02 01 05 NN F7

LPF CUTOFF F0 43 10 4C 02 01 06 NN F7

REV PAN F0 43 10 4C 02 01 0D NN F7

CHORUS TYPE F0 43 10 4C 02 01 20 41 NN F7

LFO FREQ F0 43 10 4C 02 01 22 NN F7

LFO PM DEPTH F0 43 10 4C 02 01 23 NN F7

FB LEVEL F0 43 10 4C 02 01 24 NN F7

DELAY OFSET F0 43 10 4C 02 01 25 NN F7

CHORUS PAN F0 43 10 4C 02 01 2D NN F7

SEND CHO-REV F0 43 10 4C 02 01 2E NN F7

VARIATION TYPE F0 43 10 4C 02 01 40 NN NN F7

THE FOLLOWING IS A LIST OF DATA TO ENTER INTO THE ABOVE NN NN MSB AND LSB DATA BYTES

@  :	Can be controlled by AC1 (Assignable Controller 1)

No.* :	These numbers correspond to the Parameter Suffix numbers in <Table1-3>

table ** :	Refer to “Effect Data Assign Table”

#### HALL1,2,  ROOM1,2,3,  STAGE1,2,  PLATE

No.*	Parameter	Range	Value	table**	Control

1	Reverb Time 	0.3~30.0s	0-69	table#4

2	Diffusion	0~10	0-10

3	Initial Delay	0~63	0-63	table#5

4	HPF Cutoff	Thru~8.0kHz	0-52	table#3

5	LPF Cutoff	1.0k~Thru	34-60	table#3

6

7

8

9

10	Dry/Wet	D63>W ~ D=W ~ D<W63	1-127		@

11	Rev Delay	0~63	0-63	table#5

12	Density	0~3	0-3

13	Er/ Rev Balance	E63> R~ E=R ~ E<R63	1-127

14

15	Feedback Level	-63~+63	1-127

16

#### DELAY L,C,R

No.*	Parameter	Range	Value	table**	Control

1	Lch Delay	0.1~715.0ms	1-7150

2	Rch Delay	0.1~715.0ms	1-7150

3	Cch Delay	0.1~715.0ms	1-7150

4	Feedback Delay	0.1~715.0ms	1-7150

5	Feedback Level	-63~+63	1-127

6	Cch Level	0~127	0-127

7	High Damp	0.1~1.0	1-10

8

9

10	Dry/Wet 	D63>W ~ D=W ~ D<W63	1-127		@

11

12

13	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

14	EQ Low Gain	-12~+12dB	52-76

15	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

16	EQ High Gain	-12~+12dB	52-76

#### DELAY L,R

No.*	Parameter	Range	Value	table**	Control

1	Lch Delay	0.1~715.0ms	1-7150

2	Rch Delay	0.1~715.0ms	1-7150

3	Feedback Delay 1	0.1~715.0ms	1-7150

4	Feedback Delay 2	0.1~715.0ms	1-7150

5	Feedback Level	-63~+63	1-127

6	High Damp	0.1~1.0	1-10

7

8

9

10	Dry/Wet 	D63>W ~ D=W ~ D<W63	1-127		@

11

12

13	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

14	EQ Low Gain	-12~+12dB	52-76

15	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

16	EQ High Gain	-12~+12dB	52-76

#### ECHO

No.*	Parameter	Range	Value	table**	Control

1	Lch Delay1	0.1~355.0ms	1-3550

2	Lch Feedback Level	-63~+63	1-127

3	Rch Delay1	0.1~355.0ms	1-3550

4	Rch Feedback Level	-63~+63	1-127

5	High Damp	0.1~1.0	1-10

6	Lch Delay2	0.1~355.0ms	1-3550

7	Rch Delay2	0.1~355.0ms	1-3550

8	Delay2 Level	0~127	0-127

9

10	Dry/Wet	D63>W ~ D=W ~ D<W63	1-127		@

11			12

13	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

14	EQ Low Gain	-12~+12dB	52-76

15	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

16	EQ High Gain	-12~+12dB	52-76

#### CROSS DELAY

No.*	Parameter	Range	Value	table**	Control

1	L->R Delay	0.1~355.0ms	1-3550

2	R->L Delay	0.1~355.0ms	1-3550

3	Feedback Level	-63~+63	1-127

4	Input Select	L,R,L&R	0-2

5	High Damp	0.1~1.0	1-10

6

7

8

9

10	Dry/Wet 	D63>W ~ D=W ~ D<W63	1-127		@

11

12

13	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

14	EQ Low Gain	-12~+12dB	52-76

15	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

16	EQ High Gain	-12~+12dB	52-76

#### EARLY REF1,2

No.*	Parameter	Range	Value	table**	Control

1	Type		S-H, L-H, Rdm, Rvs, Plt, Spr	0-5

2	Room Size	0.1~7.0	0-44	table#6

3	Diffusion	0~10	0-10

4	Initial Delay	0~63	0-63	table#5

5	Feedback Level	-63~+63	1-127

6	HPF Cutoff	Thru~8.0kHz	0-52

7	LPF Cutoff	1.0k~Thru	34-60

8

9

10	Dry/Wet 	D63>W ~ D=W ~ D<W63	1-127		@

11	Liveness	0~10	0-10

12	Density	0~3	0-3

13	High Damp	0.1~1.0	1-10

14

15

16

#### GATE REVERB,  REVERSE GATE

No.*	Parameter	Range	Value	table**	Control

1	Type		TypeA,TypeB	0-1

2	Room Size	0.1~7.0	0-44	table#6

3	Diffusion	0~10	0-10

4	Initial Delay	0~63	0-63	table#5

5	Feedback Level	-63~+63	1-127

6	HPF Cutoff	Thru~8.0kHz	0-52

7	LPF Cutoff	1.0k~Thru	34-60

8

9

10	Dry/Wet 	D63>W ~ D=W ~ D<W63	1-127		@

11	Liveness	0~10	0-10

12	Density	0~3	0-3

13	High Damp	0.1~1.0	1-10

14

15

16

#### CHORUS1,2,3,  CELESTE1,2,3

No.*	Parameter	Range	Value	table**	Control

1	LFO Frequency	0.00~39.7Hz	0-127	table#1

2	LFO PM Depth	0~127	0-127

3	Feedback Level	-63~+63	1-127

4	Delay Offset	0~127	0-127	table#2

5

6	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

7	EQ Low Gain	-12~+12dB	52-76

8	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

9	EQ High Gain	-12~+12dB	52-76

10	Dry/Wet 	D63>W ~ D=W ~ D<W63	1-127		@

11

12

13

14

15	Input Mode	mono/stereo	0-1

16

#### FLANGER1,2

No.*	Parameter	Range	Value	table**	Control

1	LFO Frequency	0.00~39.7Hz	0-127	table#1

2	LFO Depth	0~127	0-127

3	Feedback Level	-63~+63	1-127

4	Delay Offset	0~63	0-63	table#2

5

6	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

7	EQ Low Gain	-12~+12dB	52-76

8	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

9	EQ High Gain	-12~+12dB	52-76

10	Dry/Wet 	D63>W ~ D=W ~ D<W63	1-127		@

11

12

13

14	LFO Phase Difference	-180~+180deg	4-124	resolution=3deg.

15

16

#### SYMPHONIC

No.*	Parameter	Range	Value	table**	Control

1	LFO Frequency	0.00~39.7Hz	0-127	table#1

2	LFO Depth	0~127	0-127

3	Delay Offset	0~127	0-127	table#2

4

5

6	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

7	EQ Low Gain	-12~+12dB	52-76

8	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

9	EQ High Gain	-12~+12dB	52-76

10	Dry/Wet	D63>W ~ D=W ~ D<W63	1-127		@

11

12

13

14

15

16

#### ROTARY SPEAKER

No.*	Parameter	Range	Value	table**	Control

1	LFO Frequency	0.00~39.7Hz	0-127	table#1	@

2	LFO Depth	0~127	0-127

3

4

5

6	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

7	EQ Low Gain	-12~+12dB	52-76

8	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

9	EQ High Gain	-12~+12dB	52-76

10	Dry/Wet	D63>W ~ D=W ~ D<W63	1-127

11

12

13

14

15

16

#### TREMOLO

No.*	Parameter	Range	Value	table**	Control

1	LFO Frequency	0.00~39.7Hz	0-127	table#1	@

2	AM Depth	0~127	0-127

3	PM Depth	0~127	0-127

4

5

6	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

7	EQ Low Gain	-12~+12dB	52-76

8	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

9	EQ High Gain	-12~+12dB	52-76

10

11

12

13

14	LFO Phase Difference	-180~+180deg	4-124	resolution=3deg.

15	Input Mode	mono/stereo	0-1

16

#### AUTO PAN

No.*	Parameter	Range	Value	table**	Control

1	LFO Frequency	0.00~39.7Hz	0-127	table#1	@

2	L/R Depth	0~127	0-127

3	F/R Depth	0~127	0-127

4	PAN Direction	L<->R,L->R,L<-R,Lturn,Rturn,L/R	0-5

5

6	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

7	EQ Low Gain	-12~+12dB	52-76

8	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

9	EQ High Gain	-12~+12dB	52-76

10

11

12

13

14

15

16

#### PHASER1,2

No.*	Parameter	Range	Value	table**	Control

1	LFO Frequency	0.00~39.7Hz	0-127	table#1

2	LFO Depth	0~127	0-127

3	Phase Shift Offset	0~127	0-127

4	Feedback Level	-63~+63	1-127

5

6	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

7	EQ Low Gain	-12~+12dB	52-76

8	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

9	EQ High Gain	-12~+12dB	52-76

10	Dry/Wet	D63>W ~ D=W ~ D<W63	1-127		@

11	Stage	6~10(phaser1)/3~5(phaser2)	3-10

12	Diffusion	Mono/Stereo	0-1

13	LFO Phase Difference	-180~+180deg.	4-124	Phaser2 only

14

15

16

#### DISTORTION,  OVERDRIVE

No.*	Parameter	Range	Value	table**	Control

1	Drive		0~127	0-127		@

2	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

3	EQ Low Gain	-12~+12dB	52-76

4	LPF Cutoff	1.0k~Thru	34-60	table#3

5	Output Level	0~127	0-127

6

7	EQ Mid Frequency	500Hz~10.0kHz	28-54	table#3

8	EQ Mid Gain	-12~+12dB	52-76

9	EQ Mid Width	1.0~12.0	10-120

10	Dry/Wet 	D63>W ~ D=W ~ D<W63	1-127

11	Edge(Clip Curve) 	0~127	0-127	mild~sharp

12

13

14

15

16

#### GUITAR AMP SIMULATOR

No.*	Parameter	Range	Value	table**	Control

1	Drive		0~127	0-127		@

2	AMP Type	Off,Stack,Combo,Tube	0-3

3	LPF Cutoff	1.0k~Thru	34-60	table#3

4	Output Level	0~127	0-127

5

6

7

8

9

10	Dry/Wet	D63>W ~ D=W ~ D<W63	1-127

11	Edge(Clip Curve)	0~127	0-127	mild~sharp

12

13

14

15

16

3-BAND EQ

No.*	Parameter	Range	Value	table**	Control

1	EQ Low Gain	-12~+12dB	52-76

2	EQ Mid Frequency	500Hz~10.0kHz	28-54	table#3

3	EQ Mid Gain	-12~+12dB	52-76

4	EQ Mid Width	1.0~12.0	10-120

5	EQ High Gain	-12~+12dB	52-76

6	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

7	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

8

9

10

11

12

13

14

15

16

2-BAND EQ

No.*	Parameter	Range	Value	table**	Control

1	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

2	EQ Low Gain	-12~+12dB	52-76

3	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

4	EQ High Gain	-12~+12dB	52-76

5

6

7

8

9

10

11

12

13

14

15

16

#### AUTO WAH

No.*	Parameter	Range	Value	table**	Control

1	LFO Frequency	0.00~39.7Hz	0-127	table#1

2	LFO Depth	0~127	0-127

3	Cutoff Frequency Offset	0~127	0-127		@

4	Resonance	1.0~12.0	10-120

5

6	EQ Low Frequency	50Hz~2.0kHz	8-40	table#3

7	EQ Low Gain	-12~+12dB	52-76

8	EQ High Frequency	500Hz~16.0kHz	28-58	table#3

9	EQ High Gain	-12~+12dB	52-76

10	Dry/Wet 	D63>W ~ D=W ~ D<W63	1-127

11

12

13

14

15

16

DID WE SAY THAT THE EFFCECTS PROCESSORS WERE POWERFUL OR WHAT !!!!!!, TRY DOING THAT LOT ON ANY OTHER SOFTWARE SYSTEM IN EXISTANCE.

VARIATION CONNECTION    F0 43 10 4C 02 01 5A NN F7

THIS NEEDS TO BE SET TO SYSTEM MODE TO GET EFFECTS ON EACH MIDI CHANNEL IN THIS CASE NN NEEDS TO BE SET TO 1

DISTORTION (INS MODE)     F0 43 10 4C 03 00 00 NN 00 F7

DISTORTION PART NO         F0 43 10 4C 03 00 0C NN F7

NOTE SHIFT                        F0 43 10 4C 08 00 08 NN F7

DRUM PITCH COARSE          F0 43 10 4C 30 18 00 NN F7

DRUM PITCH FINE              F0 43 10 4C 30 18 01 NN F7

DRUM LEVEL               F0 43 10 4C 30 18 02 NN F7

DRUM PAN                  F0 43 10 4C 30 18 04 NN F7

REV SEND   DRUM        F0 43 10 4C 30 18 05 NN F7

DRUM CHO SEND           F0 43 10 4C 30 18 06 NN F7

DRUM VAR SEND         F0 43 10 4C 30 18 07 NN F7

DRUM FILTER CUTOFF  F0 43 10 4C 30 18 0B NN F7

That’s right, individual filters on every single drum sound…..cooooool !

Oh, and the  same goes for resonance, fx sends, amp envelopes and many others as well.

DRUM RESONANCE       F0 43 10 4C 30 18 0C NN F7

DRUM ALTER GROUP    F0 43 10 4C 30 18 03 NN F7

DRUM ATTACK             F0 43 10 4C 30 18 0D NN F7

DRUM EG DECAY          F0 43 10 4C 30 18 0E NN F7

DRUM DECAY 2            F0 43 10 4C 30 18 0F NN F7

##### EFFECTS ASSIGNMENT SETUP DATA TABLE

THE FOLLOWING DATA ALLOWS ENTRY OF A DATA INTO THE VARIOUS LISTED EFFECTS (SUCH AS LFO), SO THAT PRECISE EFFECTS TYPES CAN BE SET UP.

These data bytes need to been inserted within the correct parameter for each effect. For this section you may need to refer to the original manual to look at the tables for each effect type, and the parameter that you wish to edit. With the knowledge gained from this guide , this should not really be too daunting....HONEST !

Table#1

LFO Frequency (Hz)

Data	Value

0	0.00

1	0.04

2	0.08

3	0.13

4	0.17

5	0.21

6	0.25

7	0.29

8	0.34

9	0.38

10	0.42

11	0.46

12	0.51

13	0.55

14	0.59

15	0.63

16	0.67

17	0.72

18	0.76

19	0.80

20	0.84

21	0.88

22	0.93

23	0.97

24	1.01

25	1.05

26	1.09

27	1.14

28	1.18

29	1.22

30	1.26

31	1.30

32	1.35

33	1.39

34	1.43

35	1.47

36	1.51

37	1.56

38	1.60

39	1.64

40	1.68

41	1.72

42	1.77

43	1.81

44	1.85

45	1.89

46	1.94

47	1.98

48	2.02

49	2.06

50	2.10

51	2.15

52	2.19

53	2.23

54	2.27

55	2.31

56	2.36

57	2.40

58	2.44

59	2.48

60	2.52

61	2.57

62	2.61

63	2.65

64	2.69

65	2.78

66	2.86

67	2.94

68	3.03

69	3.11

70	3.20

71	3.28

72	3.37

73	3.45

74	3.53

75	3.62

76	3.70

77	3.87

78	4.04

79	4.21

80	4.37

81	4.54

82	4.71

83	4.88

84	5.05

85	5.22

86	5.38

87	5.55

88	5.72

89	6.06

90	6.39

91	6.73

92	7.07

93	7.40

94	7.74

95	8.08

96	8.41

97	8.75

98	9.08

99	9.42

100	9.76

101	10.10

102	10.80

103	11.40

104	12.10

105	12.80

106	13.50

107	14.10

108	14.80

109	15.50

110	16.20

111	16.80

112	17.50

113	18.20

114	19.50

115	20.90

116	22.20

117	23.60

118	24.90

119	26.20

120	27.60

121	28.90

122	30.30

123	31.60

124	33.00

125	34.30

126	37.00

127	39.70

Table#2

Modulation Delay Offset (ms)

Data	Value

0	0.0

1	0.1

2	0.2

3	0.3

4	0.4

5	0.5

6	0.6

7	0.7

8	0.8

9	0.9

10	1.0

11	1.1

12	1.2

13	1.3

14	1.4

15	1.5

16	1.6

17	1.7

18	1.8

19	1.9

20	2.0

21	2.1

22	2.2

23	2.3

24	2.4

25	2.5

26	2.6

27	2.7

28	2.8

29	2.9

30	3.0

31	3.1

32	3.2

33	3.3

34	3.4

35	3.5

36	3.6

37	3.7

38	3.8

39	3.9

40	4.0

41	4.1

42	4.2

43	4.3

44	4.4

45	4.5

46	4.6

47	4.7

48	4.8

49	4.9

50	5.0

51	5.1

52	5.2

53	5.3

54	5.4

55	5.5

56	5.6

57	5.7

58	5.8

59	5.9

60	6.0

61	6.1

62	6.2

63	6.3

64	6.4

65	6.5

66	6.6

67	6.7

68	6.8

69	6.9

70	7.0

71	7.1

72	7.2

73	7.3

74	7.4

75	7.5

76	7.6

77	7.7

78	7.8

79	7.9

80	8.0

81	8.1

82	8.2

83	8.3

84	8.4

85	8.5

86	8.6

87	8.7

88	8.8

89	8.9

90	9.0

91	9.1

92	8.2

93	9.3

94	9.4

95	9.5

96	9.6

97	9.7

98	9.8

99	9.9

100	10.0

101	11.1

102	12.2

103	13.3

104	14.4

105	15.5

106	17.1

107	18.6

108	20.2

109	21.8

110	23.3

111	24.9

112	26.5

113	28.0

114	29.6

115	31.2

116	32.8

117	34.3

118	35.9

119	37.5

120	39.0

121	40.6

122	42.2

123	43.7

124	45.3

125	46.9

126	48.4

127	50.0

Table#3

EQ Frequency (Hz)

Data	Value

0	THRU(20)

1	22

2	25

3	28

4	32

5	36

6	40

7	45

8	50

9	56

10	63

11	70

12	80

13	90

14	100

15	110

16	125

17	140

18	160

19	180

20	200

21	225

22	250

23	280

24	315

25	355

26	400

27	450

28	500

29	560

30	630

31	700

32	800

33	900

34	1.0k

35	1.1k

36	1.2k

37	1.4k

38	1.6k

39	1.8k

40	2.0k

41	2.2k

42	2.5k

43	2.8k

44	3.2k

45	3.6k

46	4.0k

47	4.5k

48	5.0k

49	5.6k

50	6.3k

51	7.0k

52	8.0k

53	9.0k

54	10.0k

55	11.0k

56	12.0k

57	14.0k

58	16.0k

59	18.0k

60	THRU(20.0k)

Table#4

Reverb Time (s)

Data	Value

0	0.3

1	0.4

2	0.5

3	0.6

4	0.7

5	0.8

6	0.9

7	1.0

8	1.1

9	1.2

10	1.3

11	1.4

12	1.5

13	1.6

14	1.7

15	1.8

16	1.9

17	2.0

18	2.1

19	2.2

20	2.3

21	2.4

22	2.5

23	2.6

24	2.7

25	2.8

26	2.9

27	3.0

28	3.1

29	3.2

30	3.3

31	3.4

32	3.5

33	3.6

34	3.7

35	3.8

36	3.9

37	4.0

38	4.1

39	4.2

40	4.3

41	4.4

42	4.5

43	4.6

44	4.7

45	4.8

46	4.9

47	5.0

48	5.5

49	6.0

50	6.5

51	7.0

52	7.5

53	8.0

54	8.5

55	9.0

56	9.5

57	10.0

58	11.0

59	12.0

60	13.0

61	14.0

62	15.0

63	16.0

64	17.0

65	18.0

66	19.0

67	20.0

68	25.0

69	30.0

Table#5

Delay Time (ms)

Data	Value

0	0.1

1	1.7

2	3.2

3	4.8

4	6.4

5	8.0

6	9.5

7	11.1

8	12.7

9	14.3

10	15.8

11	17.4

12	19.0

13	20.6

14	22.1

15	23.7

16	25.3

17	26.9

18	28.4

19	30.0

20	31.6

21	33.2

22	34.7

23	36.3

24	37.9

25	39.5

26	41.0

27	42.6

28	44.2

29	45.7

30	47.3

31	48.9

32	50.5

33	52.0

34	53.6

35	55.2

36	56.8

37	58.3

38	59.9

39	61.5

40	63.1

41	64.6

42	66.2

43	67.8

44	69.4

45	70.9

46	72.5

47	74.1

48	75.7

49	77.2

50	78.8

51	80.4

52	81.9

53	83.5

54	85.1

55	86.7

56	88.2

57	89.8

58	91.4

59	93.0

60	94.5

61	96.1

62	97.7

63	99.3

64	100.8

65	102.4

66	104.0

67	105.6

68	107.1

69	108.7

70	110.3

71	111.9

72	113.4

73	115.0

74	116.6

75	118.2

76	119.7

77	121.3

78	122.9

79	124.4

80	126.0

81	127.6

82	129.2

83	130.7

84	132.3

85	133.9

86	135.5

87	137.0

88	138.6

89	140.2

90	141.8

91	143.3

92	144.9

93	146.5

94	148.1

95	149.6

96	151.2

97	152.8

98	154.4

99	155.9

100	157.5

101	159.1

102	160.6

103	162.2

104	163.8

105	165.4

106	166.9

107	168.5

108	170.1

109	171.7

110	173.2

111	174.8

112	176.4

113	178.0

114	179.5

115	181.1

116	182.7

117	184.3

118	185.8

119	187.4

120	189.0

121	190.6

122	192.1

123	193.7

124	195.3

125	196.9

126	198.4

127	200.0

Table#6

Room Size (m)

Data	Value

0	0.1

1	0.3

2	0.4

3	0.6

4	0.7

5	0.9

6	1.0

7	1.2

8	1.4

9	1.5

10	1.7

11	1.8

12	2.0

13	2.1

14	2.3

15	2.5

16	2.6

17	2.8

18	2.9

19	3.1

20	3.2

21	3.4

22	3.5

23	3.7

24	3.9

25	4.0

26	4.2

27	4.3

28	4.5

29	4.6

30	4.8

31	5.0

32	5.1

33	5.3

34	5.4

35	5.6

36	5.7

37	5.9

38	6.1

39	6.2

40	6.4

41	6.5

42	6.7

43	6.8

44	7.0

#### TURNING OFF CHANNELS ON THE SYXG50

With the SYXG50 having  16 parts and up to 128 note polyphony, and each part having an assignable midi channel individual parts can be switched off. To do this requires the use of a system exclusive message.

##### THE MESSAGE IS AS FOLLOWS

#### F0 43 10 4C 08 NN 04 7F F7

Where nn is the part number, or midi channel no. (EG 01 - 0F)

#### NRPN’S - WHAT ARE THEY ???

You may have heard about the term NRPN, this stands for NON REGISTERED PARAMETER NUMBER. These can be use for specific things such as filter cutoff or for pitch changes, and are espacially useful for changing drum sounds,for example to create booming kick drums or fizzy hi hats.

NRPN entry requires several data bytes to be fed into you sequencer event list, the controller messages are 99 and 98 in that order, I will now give a worked example for changing the pitch of the kick drum.

CONTROLLER VALUE WHAT IT DOES

99 24 TELLS THE DB50 THAT PITCH IS TO BE CHANGED

98 36 TELLS THE DB50 THAT SOUND 36 (KICK) IS THE 1

6 NN ENTER YOU OWN DAT FOR THE PITCH OF THE KICK

IN TG300 MODE YOU NEED TO TELL THE SYSTEM TO ENABLE NRPN’S ON DRUM SOUNDS. TO DO THIS SEND THE FOLLOWING SYSEX.

#### F0 41 10 56 12 01 00 01 01 7D F7

FUN FUN FUN  !!!!

#### CHAPTER 5 : SEQUENCER SPECIFICS

#### USING THE ABOVE IN TWELVE TONE SYSTEMS CAKEWALK

These events should be entered as system exclusive banks in the Cakewalk program. If you select VIEW from the options menu at the top of the screen then option to edit and display sysex. The user then inserts the sysex stream of data and labels it with either a name or a number.

This number is then inserted into the event list, by opening up the event list page and hitting the insert key, and selecting SYSEX as the event type. The value of the number associated with the sysex byte is then inserted.

##### A similar procedure is used for POWER TRACKS PRO

CUBASE allows direct entry of the sysex stream into the event list, in a similar way to Cakewalk, but in the case of Cubase, the data is entered not as a single number but as a complete sysex stream.

#### EG

F0 7E 7F 09 01 F7 THIS IS GM ON

F0 43 10 4C 00 00 7E 00 F7 THIS IS XG ON

As a rule of thumb it is a very good idea at the beginning of all of your sequncer tracks to have a GM on command first, followed by an XG on command 200 milliseconds later. This is the time determined by the XG specs, and should really be adhered to if you want to avoid problems.

#### THE USE OF CONTROLLERS

Controller information allows real-time expression to be added to a track, whether it be a filter sweep or resonance, or a modification to attack time.

XG protocol allows much more expressive real-time control than ever before, with multiple simultaneous controller information allowed

A list of controllers that may be applied to any XG compatible device can be found in the XG SPEC level 1. This can be downloaded from

www.Yamaha.co.uk

#### EFFECTS PROCESSING APPLICATIONS

As mentioned before, the SYXG50 has 3 independent effects processors each of which can have their own send and return levels, along with cross modulation options. By cross modulation I mean that the output from one effect processor can be routed to another, i.e you can send chorus to reverb or variation to reverb to create a different effect on each channel.

Send and return levels can be easily explained by imagining a floor effect pedal such as those used by guitar players. If they wish to use this in conjunction with an amplifier they would normally route this to the effect send on the back of the amp. The amp would then have a rotary pot on the front which defined the send level, or amount of the amplified main guitar sound being sent to the effect processor. With mixing desks you also have the luxury of a return level , so that you can decide how much of the effect is then sent back into the main signal path.

The options supported by SYXG50 are listed below

CHORUS SEND LEVEL

CHORUS TO REVERB SEND

VARIATION SEND LEVEL

VARIATION TO CHORUS SEND LEVEL

VARIATION TO REVERB SEND LEVEL

REVERB SEND LEVEL

CHORUS RETURN LEVEL

VARIATION RETURN LEVEL

REVERB RETURN LEVEL

All of the above can be set independently for each of the midi channels, giving you more than the standard 3 buss options.

SYXG50 is also the first software wavetable set to respond to polyphonic after touch, this facility allows individual expression to be applied to individual keys ( i.e. adding filter resonance to the top note in a chord sequence). Most keyboards on the market cannot send out POLY KEY, one notable exception is the Ensoniq range (SQ80,EPS,TS,VFX,AND ASR)

Polyphonic aftertouch allows subtleties to be added to an instrumental solo that was never before possible on a sound card.

APPLICATION OF CONTROLLER INFORMATION IN REAL-TIME USING TWELVE TONE SYSTEMS CAKEWALK SOFTWARE.

The ability within the Cakewalk environment to add controller information is not unique, but the way in which it can be applied from the cakewalk fader page is very easy.

As mentioned before the DB50 responds to a host of controller messages. Modifications to filters, chorus levels, resonance etc etc can be sent out in real time from even a basic sequencer package with ease.

A controller message is exactly as it sounds, it allows control of a specific parameter. Controllers may either be drawn in using the pencil tool or added in real time using the definable fader page.

On selection of the fader page within Cakewalk Pro , the user can then define either a rotary control pot or a fader page to any controller number, a group function allows multiple controllers to be assigned at any one time and moved simultaneously.

The Controllers view page allows the user to draw in manual corrections to the data that has been fed in via the fader page , or to draw in accurate control applications such as filter cutoff.

Above you can sse a typical track setup within the event list editor on cakewalk

#### CUBASE

User definable mixer maps allow complex sysex and controller information to be applied within the Cubase environment. A full description on this can be found in the Cubase user manual.

It is by far the most flexible system for controller and sysex information on any sequencer platform, allowing the user to define either a fader, knob , or rotary pot, and then apply any data in effect to this .

This feature is only found in Cubase score, Cubase Audio, but not in Cubase Compact, Lite or Cubasis.

#### SETTING UP YOUR MAGNUM OPUS SEQUENCER TRACK

So, you now have all of the tools to do the job, how should you go about constructing it all. Well propably the best way is to follow this simple set of rules for your setup track.

1: Insert SMF meta events such as the name of the track, tempo, time signature etc (See your sequencer manual for how to do this )

2: Insert the GM system on message (sysex)

3: Leave at least a 200millisecond gap

4: Now insert an XG on system message.

5: Leave at least a 50 millisecond gap

6: Now send all the system exclusive messages for the effects (variation connection type, variation type, and the effects parameters...in that order)

7: All of the bank select messages for each midi channel, followed by all of the program change messages.

8: All of the required controller messages

9 : All of the RPN and NRPN messages for your song (drums, filters etc)

10 : Repeat the steps 7-11 for all 16 midi channels

For a reference please examine the midi files that can be found at the Yamaha.co.uk website.

An easier way of doing all of the above is to download XGEDIT, try it, if you like it (we do) then register it with the programmer GARY GREGSON, this will enable you to edit your SYXG50 easily and then paste the edits into your sequencer.

#### CHAPTER 6 : USING THE SYXG50 WITH SPECIFIC SOUND CARDS.

Things to look out for.

1 : The Creative Labs SB16 pro edition/ MULITCD/ ASP/SCSI2

No real problems here, just install and go. Make sure that you are not trying to run the Creative Netsynth/Wavesynth at the same time though…(why would you ??)

2: The Creative Labs AWE32/64/64 Gold .

As above really, just make sure that the WS/WG upgrade that you may have installed is OFF (just do an A/b comparison, and your will want to turn it off, and probably never use it again anyway !)

3: The Turtle Beach Tahiti/ Pinnacle, a real sound card with some class.  (the one we all use here at Yamaha UK)

One of the most popular range of cards for professional audio use, as it features high quality AD/DA converters and will handle simultaneous record and playback properly . Again just a matter of Installing the SYXG50. Instant wavetable for a card that has none (in the case of the Tahiti). The card is not however games compatible in DOS, and as yet has no direct sound support.(only really applies to games users).

#### THE YAMAHA SW60 ,ISA  XG CARD

This is not a wave playback card in the sense of the standard soundblaster raneg, as it has no CODEC. This card is generally used in conjunction with a SB16 or similar. So not only can you have the 32 note polyphony of the SW60XG, but also the 128 note polyphony of SYXG50.

#### SOFTWARE

Yamaha have a great relationship with many of the XG community . One of the best people we have had the fortune of working with is Mr Gary Gregson, who wrote a very nice program for editing XG units. This program known as XGEDIT allows full access to the SYXG50, and in the registered version allows full saving of all sysex parameters in either midi file format or Cakewalk .syx format.

The shareware version of the program is available free of charge from Yamaha.co.uk, with the registered version being available from the programmer , Gary Gregson at a cost of £25. His details are to be found within the program itself.

The editor allows the user who has no prior knowledge of sysex or controllers to see in a form which he may be more familiar with, all of the functions of the SYXG50 Which is without doubt the world’s most powerful software synth.

XGEDIT technical support questions should be referred to Gary Gregson, and not to Yamaha.

#### SOME COOL NET SITES :

WWW.YAMAHA.CO.UK

WWW.YAMAHA.COM

WWW.YAMAHA.CO.JP/ENGLISH

#### SO IN SUMMARY

1: THE SYXG50 IS THE MOST POWERFUL SOFTWARE SYNTH  IN THE WORLD (TRY COMPARING ANY OTHER TO IT AND WATCH IT CRUMBLE !!!)

2: DON’T TRY RUNNING IT AT FULL PELT ON A P90, IT WONT HAPPEN

If you have a slow machine then all of the Yamaha XG cards (SW60/DB50 will work happily on your machines)

(We are not running what intel are saying is below entry level..so why should you)

3: XG EDIT IS AVAILABLE FROM GARY GREGSON FOR £25 (And is well worth it)

4: INSTALLATION OF SYXG50 IS REALLY EASY, JUST REMEMBER NOT TO DELETE THE FOLDER, BUT TO DE-INSTALL USING THE UNINSTALL UTILITY, IF YOU ARE UPGRADING FROM AN EARLIER YAMAHA SOFTSYNTH..

5: COMPUTERS ARE SAFE (EXCEPT IF YOU POUR COLD WATER ON THEM)

6: THE SYXG50 IS APPROXIMATELY £500 CHEAPER THAN ANY OF IT’S COMPETITORS (HA...HA !...WHAT COMPETITORS ??)

7: HAVE FUN ... AND MAKE SOME GOOD MUSIC

##### YAMAHA XG

##### REVIEWS ROUNDUP

###### PC FORMAT - FEB 1996 - BEST NEW HARDWARE AWARD - DB50XG

###### “THE SOUND IS SUCH GOOD QUALITY THAT WE COULDN’T HELP BUT HEAP AWARDS ON IT”

###### COMPUTER ARTS MAGAZINE - DEC 95 - 5 STAR AWARD

###### “ THE AWESOME POWER AND FLEXIBILITY OF XG MAKES IT THE CONNOISERS CHOICE “

###### “STREETS AHEAD OF THE AWE32” (The 64 as well…we know)

###### “COMPLETELY GOBSMACKED BY THE SOUND “

###### COMPUTER LIFE MAGAZINE - FEB 1996 - 5 STAR AWARD

###### “IF YOU NEED HIGH QUALITY SOUND ON YOUR PC , YOUR SEARCH IS OVER”

###### “TRANSFORMS GAMES MUSIC INTO CINEMATIC EPICS”

###### “MUSICIANS WILL JUST DROOL OVER IT’S CAPABILITIES”

###### PC FORMAT - DEC 95 - GOLD AWARD

###### “THIS IS A MUSICIANS WET DREAM”

###### “DEVESTATES THE AWE32 AND ALL OTHER CARDS IN THIS TEST”

###### “THIS IS THE BUSINESS”

###### “AWESOME POWER”

###### THE MIX MAGAZINE - 96% - HIGHEST SCORE EVER FOR A SOUND CARD

###### “THE SOUNDS ARE SUPERB”

###### “I’M RUNNING OUT OF SUPERLATIVES TO DESCRIBE THIS CARD”

###### “GO OUT AND BUY ONE”

###### PERSONAL COMPUTER WORLD - DEC95 - 5 STAR AWARD

###### “COMPLETELY DESTROYS THE COMPETITION”

“SELL YOUR GRANDMOTHER OR ROB A BANK- JUST DO WHAT IS NECESSARY TO GET THIS”

###### “THE BEST SOUNDING PC SOUND KNOWN TO MAN”

###### MULTIMEDIA WORLD - NOV95 - 5 STAR AWARD

###### “THE MOST IMPRESSIVE ADD ON WE HAVE EVER HEARD”

###### “THE SOUND IS SO REALISTIC YOU THINK YOU ARE LISTENING TO AN AUDIO CD”

###### EQ MAGAZINE

“ I DID SOME COMPARISONS BETWEEN THE XG AND THE ROLAND SCD15/SCD10 AND VARIOUS OTHER CARDS... THE XG SYSTEMS LEFT THEM FOR DUST”

###### PC FORMAT - FEB 1996- PLATINUM AWARD WINNER

###### “WHAT A SOUND”

###### “TOP FLIGHT SYNTH PERFORMANCE”

###### “IT IS SIMPLY SUPERB”

###### “UNBELIEVABLE PRICE”

###### “THE CATCH IS .. THERE IS NO CATCH”

###### WINDOWS PLUS MARCH 1996

“LIKE STRAPPING A ROCKET BOOSTER TO YOUR CAR, IT HAS THE SAME KIND OF EFFECT WITH YOUR EXISTING SOUND CARD”

# Glosaary

Some of the terms used in this guide may be a bit confusing, here is a summary of them.

## Chorus

An effect which adds depth and thickness to musical sound. This is a system effect (it applies to the overall sound).

#### CODEC

A hardware system consisting of A/D (Analog/Digital) converter, D/A (Digital/Analog) converter, and analog mixer which is indispensable in multimedia computers. Converts the digital audio data generated by the software synthesizer to analog signals for output as sound.

## CPU Load

A parameter which determines how much processing load the software synthesizer places on the computer’s CPU. Reducing the value of this parameter reduces the amount of processing power the computer assigns to the software synthesizer, thereby reducing the number of simultaneous notes which can be produced.

## Direct Sound

A part of Microsoft’s DirectX system which dramatically increases hardware efficiency. Normally, a single wave device driver is used to produce sound. If the Direct Sound driver is installed, however, Direct Sound functions as if there were several wave device drivers producing different frequencies, panning waveforms, etc., for games.

## Drag

## Moving the computer’s on-screen cursor to a different location while holding the mouse button.

## Drop

## The act of releasing the mouse button after “dragging”

## Effect

Effects allow the sound to be varied in a number of ways for greater ambiance, animation, and/or expression. The S-YXG50 has three main effect groups: Reverb, Chorus, and Variation.

## Filter

A filter can remove or emphasize certain parts of the frequency spectrum of a sound to change the overall timbre. The S-YXG50 includes a time-variable digital filter.

## GM System Level 1

This is a standard tone generator voice assignment agreed upon by most musical instrument and software manufacturers that allows MIDI music data to be played back on just about any tone generator with predictable results.

#### MIDI

MIDI, the Musical Instrument Digital Interface, is a world-standard communication interface that allows MIDI-compatible musical instruments and equipment to share musical information and control one another. This makes it possible to create “systems” of MIDI instruments and equipment that offer far greater versatility and control than is available with isolated instruments.

## MMX ™ Technology

MMX ™ technology significantly increases CPU processing speed, allowing many functions which could previously only be handled using dedicated hardware to be comfortably handled using only software. XG compatible software synthesizers have been made possible thanks to MMX ™ technology.

## MMX ™ Technology Pentium Processor

## A new Pentium CPU based on Intel’s MMX ™ technology.

## Polyphony

The maximum polyphony sounded at one time using the soft synthesizer. The maximum polyphony  to be played simultaneously depend on the computer CPU you use.

## Reverb

An effect which simulates the natural reverberation of a “live” acoustic environment such as a hall or room. In the S-YXG50 Reverb is a system effect (i.e. it applies to the overall sound).

## Sampling Rate

This important parameter determines playback sound quality. a 44 kHz sampling rate produces CD-quality sound, 22 kHz is radio quality, and 11 kHz is approximately telephone quality.

## Soft Synthesizer

A synthesizer (tone generator), in which the sound is produced entirely using software (without hardware tone generator), to play back MIDI song files. Song playback is performed using audio function built into the PC. The high performance CPU has made it possible to play back the sounds without external tone generator.

## Standard MIDI File

A MIDI data format which provides playback compatibility between a wide range of computers and electronic musical instruments. Standard MIDI files can be played using any SMF (Standard MIDI File) compatible software application or musical instrument. Sometimes simply called a “MIDI file”.

## Variation

In the S-YXG50 this refers to an effect group which includes reverb, chorus, delay, distortion, and others. Variation effects can be used as system effects (they apply to the overall sound) or insertion effects (the effect can be applied to specific parts).

## XG Format

An original Yamaha extension to the GM System Level 1 specifications, providing greater expressive capability, enhanced sound control, and plenty of room for future expansion while maintaining data compatibility. XG-compatible tone generators and music software bear the Yamaha XG logo.

### MANY THANKS FOR YOUR SUPPORT ON XG AND SYXG50, WE REALLY HOPE THAT , LIKE EVERY MAG SO FAR, YOU THINK THAT XG IS SUPERB AND A GREAT LEAP FORWARD OVER THE MUNDANE JUNK THAT CERTAIN OTHER SOUND CARD COMPANIES KEEP CHURNING OUT AND RE-NAMING.

Coming soon….. The SYXG50 Lion King pack (Just Kidding !)

AND BELIEVE US….IT WONT STOP HERE…

ALL COMPANY TRADE NAMES AND TRADE MARKS CONTAINED WITHIN THIS DOCUMENT ARE NOTED.

Sections of this document were taken from threads on Compuserve, and we GRATEFULLY acknowledge the contribution of the authors.

## Sound Blaster and Waveblaster are a trade mark of Creative Labs.(yes..well…great…..hmmm!)

MICROSOFT WINDOWS 95 IS A TRADEMARK OF MICROSOFT CORPORATION (WHILST IT IS A MAJOR LEAP ON WINDOWS 3.1, ALL I CAN SAY IS…ROLL ON WINDOWS 97..IT LOOKS GREAT…AND NT 5.0 (WE LOVE NT 4.0) (TOTALLY COOL O.S)

MMX IS A TRADEMARK OF INTEL CORP (AND A KILLER SYSTEM IT IS TOO)

LION KING IS COPYRIGHT DISNEY (AND A GREAT FILM)

ALL OTHER TRADEMARKS ARE THE PROPERTY OF THEIR RESPECTIVE OWNERS.